- A sequence x[n] has a DFT X[k] = {1, 2, 1, 0}. If a new sequence y[n] is created by a circular time shift of x[n] by 1 unit (y[n] = x[n-1]), find the DFT of y[n], Y[k].

- The DFT of x(n) is X(k) = {5,  3+j,  1,  3−j}, find the DFT of x(−n)

- If x[n] has a DFT X[k] = {4, 0, 0, 0}, what is the DFT of the time-reversed sequence x[-n]?

- A sequence x[n] has a DFT X[k]. If y[n] = x[n] * cos(πn/2), find the DFT of y[n]. Assume 4 point DFT.

- The first five points of the eight point DFT of a real valued sequence are {0.25, 0.125- j0.3018, 0, 0.125- j0.0518, 0, …………..} Determine the remaining three points.

- Compute the circular convolution between the following sequences using DFT and IDFT method x(n)={1,2,3,4}, y(n)={−1  ,−2,−3,−4} where x(n) and y(n)are periodic sequences with period n=4.

- If x(n)= {2,9,0,4}. Find X(k), X(K-2), X(k+2), X(-k)

- Take any sequence for N=4. Find its DFT. Now insert alternate zeros in the sequence. Find the DFT of the resultant sequence of order N=8. What do you observe.

- If x(n)={1,0,2,0,3,0,4,0}. For N=8, Find X(k), X(-k), X(-k+3). Find x(n+2), x(n-1).

- If X(k)={15, 2-5j, -11, 2+5j}. Find y(n) for Y(k)=X(k-4).

- For each of the above questions, verify the Parsevals theorem.

- Write the procedure to perform Linear convolution.

- How to solve Linear using circular convolution.

- Determine the output response y(n) if h(n)= {1,1,1}; x(n)= {1,2,3,1}by using i)Linear Convolution ii) Circular convolution iii) Linear convolution using circular convolution

- Find output of the system defined as h(n)={1,2} for given input x(n)= {1,2,3} using DFT/IDFT.

- Find output of the system defined as h(n)={1,2} for given input x(n)= {1,2,3} using circular convolution.

- x1(n) = {1, 2, 3}, x2(n) = {3, 2, 1}. Find x1(n) ⊛ x2(n) (3-point circular convolution) using DFT.

- The DFT of x(n) is X(k)= {8,  4,  2,  0},find the DFT of y(n)=(−1)n x(n)

- The DFTs of two sequences are X(k)={4,  2,  0,  2}, H(k)={1, j,−1,−j} ,Find the DFT of y(n)=x(n)⊛h(n)

- Verify Parseval’s theorem for x(n)={1,2,1,0}

- The DFT of x[n] is  X(k) = {10, -2+j2, -2, -2-j2}, N=4. Find DFT of

y(n) = x(n)·e^(j2πln/N) for l = 1